/*!

\page about_hypo_page About Hypotheses

\n \b Hypotheses represent boundary conditions which will be taken into

account at calculations of meshes or submeshes basing on geometrical

objects. These hypotheses allow you to manage the level of detail of

the resulting meshes or submeshes: when applying different hypotheses

with different parameters you can preset the quantity of meshing

elements which will compose your mesh. So, it will be possible to

generate a rough or a more refined mesh or submesh.

In \b MESH there are the following Basic Hypotheses (to introduce

them, you operate numerical values):

<ul>

<li>\subpage a1d_meshing_hypo_page "1D Hypotheses" (for meshing of

<b>edges</b>):</li>

<ul>

<li>\ref arithmetic_1d_anchor "Arithmetic 1D"</li>

<li>\ref average_length_anchor "Average Length"</li>

<li>\ref max_length_anchor "Max Size"</li>

<li>\ref deflection_1d_anchor "Deflection 1D"</li>

<li>\ref number_of_segments_anchor "Number of segments"</li>

<li>\ref start_and_end_length_anchor "Start and end length"</li>

<li>\ref automatic_length_anchor "Automatic Length"</li>

</ul>

<li>\subpage a2d_meshing_hypo_page "2D Hypotheses" (for meshing of <b>faces</b>):</li>

<ul>

<li>\ref max_element_area_anchor "Max Element Area"</li>

<li>\ref length_from_edges_anchor "Length from Edges"</li>

<li>\ref hypo_quad_params_anchor "Quadrangle Parameters"</li>

<li>\ref quadrangle_preference_anchor "Quadrangle Preference"</li>

<li>\ref triangle_preference_anchor "Triangle Preference"</li>

</ul>

<li>3D Hypothesis (for meshing of <b>volumes</b>):</li>

<ul>

<li>\subpage max_element_volume_hypo_page "Max Element Volume"</li>

</ul>

</ul>

Some hypotheses are strictly combined with plug-in

meshers and thus, work only with definite algorithms.

<ul>

<li>\subpage netgen_2d_3d_hypo_page</li> - work with NetGen algorithm.

<li>\subpage ghs3d_hypo_page</li> - works with GHS3D algorithm.

<li>\subpage ghs3dprl_hypo_page</li> - works with GHS3DPRL (tepal) algorithm.

<li>\subpage blsurf_hypo_page</li> - works with BLSURF algorithm.

</ul>

There also exist

\subpage additional_hypo_page "Additional Hypotheses" used together

with other hypotheses:

<ul>

<li>Propagation of 1D Hypothesis on opposite edges</li>

<li>Non conform mesh allowed</li>

<li>Quadratic mesh</li>

</ul>

The choice of a hypothesis depends on:

<ul>

<li>the geometrical object (shape) which will be meshed</li>

<li>the algorithm, which will be selected for meshing of this geometrical object (shape)</li>

</ul>

*/