/*!

\page blsurf_hypo_page BLSURF Parameters hypothesis

\n BLSURF Parameters hypothesis works only with <b>BLSURF</b> 2d

algorithm. This algorithm is a commercial software.

<h1>General parameters</h1>

\image html blsurf_parameters.png

<ul>

<li><b>Name</b> - allows defining the name of the hypothesis (BLSURF

Parameters_n by default).</li>

<li><b>Physical Mesh</b> - can be set to None, Custom or Size Map

<ul>

<li>if set to "Custom", allows user input in the

in <b>User size</b>, <b>Max Physical Size</b> and <b>Min Physical

Size</b> fields.</li>

<li>if set to "Size Map", behaves like "Custom" mode and takes into account the custom elements sizes given in the Size Map tab.</li>

</ul>

</li>

<li><b>User size</b> - defines the size of the generated mesh elements. </li>

<li><b>Max Physical Size</b> - defines the upper limit of mesh element size. </li>

<li><b>Min Physical Size</b> - defines the lower limit of mesh element size. </li>

<li><b>Geometrical mesh</b> - if set to "Custom", allows user input in

<b>Angle Mesh S</b>, <b>Angle Mesh C</b> and

<b>Gradation</b> fields. These fields control

computation of the element size, so called <i>geometrical size</i>, conform to

the surface geometry considering local curvatures. \n

If both the <b>User size</b> and the <i>geometrical size</i> are defined, the

eventual element size correspond to the least of the two. </li>

<li><b>Angle Mesh S</b> - maximum angle between the mesh face and the

tangent to the geometrical surface at each mesh node, in degrees. </li>

<li><b>Angle Mesh C</b> - maximum angle between the mesh edge and the

tangent to the geometrical curve at each mesh node, in degrees. </li>

<li><b>Max Geometrical Size</b> - defines the upper limit of the <i>geometrical size</i>.</li>

<li><b>Min Geometrical Size</b> - defines the lower limit of the <i>geometrical size</i>.</li>

<li><b>Gradation</b> - maximum ratio between the lengths of

two adjacent edges. </li>

<li><b>Allow Quadrangles</b> - if checked, allows the creation of quadrilateral elements.</li>

<li><b>Patch independent</b> - if checked, geometrical

edges are not respected and all geometrical faces are meshed as one

hyper-face.</li>

<h1>Advanced parameters</h1>

\image html blsurf_parameters_advanced.png

<li><b>Topology</b> - allows creation of a conform mesh on a shell of

not sewed faces.

<ul>

<li>"From CAD" means that mesh conformity is assured by conformity

of a shape.</li>

<li>"Pre-process" and "Pre-process++" allow the BLSURF software to

pre-process the geometrical model to eventually produce a conform

mesh. </li>

</ul>

<li><b>Verbosity level</b> - Defines the percentage of "verbosity" of

BLSURF [0-100].</li>

<li><b>Add option</b> - provides the choice of multiple advanced

options, which appear, if selected, in a table where it is possible to

input the value of the option and to edit it later.</li>

<li><b>Clear option</b> - removes the option selected in the table.

</ul>

\n

The following options are commonly usable. The notion of <i>diag</i>

used in the descriptions means

the diagonal of the bounding box of the geometrical object to mesh.

<ul>

<li><b>topo_eps1</b> (real) - is the tolerance level inside a CAD

patch. By default is equal to <i>diag</i> � 10-4. This tolerance is used to

identify nodes to merge within one geometrical face when \b Topology

option is to pre-process. Default is <i>diag</i>/10.0.</li>

<li><b>topo_eps2</b> (real) - is the tolerance level between two CAD

patches. By default is equal to <i>diag</i> � 10-4. This tolerance is used to

identify nodes to merge over different geometrical faces when

\b Topology option is to pre-process. Default is <i>diag</i>/10.0.</li>

<li>\b LSS (real) - is an abbreviation for "length of sub-segment". It is

a maximal allowed length of a mesh edge. Default is 0.5.</li>

<li>\b frontal (integer)

<ul>

<li> 1 - the mesh generator inserts points with an advancing front method.</li>

<li> 0 - it inserts them with an algebraic method (on internal edges). This method is

slightly faster but generates less regular meshes. </li>

</ul>

Default is 0.</li>

\anchor blsurf_hinterpol_flag

<li>\b hinterpol_flag (integer) - determines the computation of an

interpolated value <i>v</i> between two points <i>P1</i> and <i>P2</i> on a

curve. Let <i>h1</i> be the value at point <i>P1,</i> <i>h2</i> be the value at point

<i>P2,</i> and <i>t</i> be a parameter varying from 0 to 1 when moving from <i>P1

to</i> <i>P2</i> .

<ul>

<li>0 - the interpolation is linear: <i>v = h1 + t (h2 - h1 )</i></li>

<li>1 - the interpolation is geometric: <i>v = h1 * pow( h2/h1, t)</i></li>

<li>2 - the interpolation is sinusoidal: <i>v = (h1+h2)/2 +

(h1-h2)/2*cos(PI*t)</i></li>

</ul>

Default is 0.</li>

\anchor blsurf_hmean_flag

<li>\b hmean_flag (integer) - determines the computation of the average of several

values:<ul>

<li>-1 - the minimum is computed.</li>

<li>0 or 2 - the arithmetic average computed.

<li>1 - the geometric average is computed.</li>

</ul>

Default is 0.</li>

<li>\b CheckAdjacentEdges, \b CheckCloseEdges and \b CheckWellDefined

(integers) - gives the number of calls of equally named subroutines the

purpose of which is to improve the mesh of domains having narrow

parts. At each iteration,\b CheckCloseEdges decreases the sizes of the

edges when two boundary curves are neighboring,\b CheckAdjacentEdges

balances the sizes of adjacent edges, and \b CheckWellDefined checks if

the parametric domain is well defined. Default values are 0.</li>

<li>\b CoefRectangle (real)- defines the relative thickness of the rectangles

used by subroutine \b CheckCloseEdges (see above). Default is 0.25.</li>

<li>\b eps_collapse (real) - if more than 0.0, BLSURF removes

curves whose lengths are less than \b eps_collapse. To obtain an

approximate value of the length of a curve, it is arbitrarily

split into 20 edges. Default is 0.0.</li>

<li>\b eps_ends (real) - is used to detect the curves whose lengths are very

small, which sometimes constitutes an error. A message is printed

if<i> fabs(P2-P1) < eps_ends</i>, where <i>P1</i> and <i>P2</i> are the

extremities of a curve. Default is <i>diag</i>/500.0.</li>

<li>\b prefix (char) - is a prefix of the files generated by

BLSURF. Default is "x".</li>

<li>\b refs (integer) - reference of a surface, used when exporting

files. Default is 1.</li>

</ul>

\n

The following advanced options are not documented and you can use them

at your own risk.

\n\n Integer variables:

<ul>

<li>    addsurf_ivertex</li>

<li>    background     </li>

<li>    coiter         </li>

<li>    communication  </li>

<li>    decim          </li>

<li>    export_flag    </li>

<li>    file_h         </li>

<li>    gridnu         </li>

<li>    gridnv         </li>

<li>    intermedfile   </li>

<li>    memory         </li>

<li>    normals        </li>

<li>    optim          </li>

<li>    pardom_flag    </li>

<li>    pinch          </li>

<li>    rigid          </li>

<li>    surforient     </li>

<li>    tconf          </li>

<li>    topo_collapse  </li>

</ul>

Real variables:

<ul>

<li>    addsurf_angle  </li>

<li>    addsurf_R      </li>

<li>    addsurf_H      </li>

<li>    addsurf_FG     </li>

<li>    addsurf_r      </li>

<li>    addsurf_PA     </li>

<li>    angle_compcurv </li>

<li>    angle_ridge    </li>

<li>    eps_pardom     </li>

</ul>

String variables:

<ul>

<li>    export_format  </li>

<li>    export_option  </li>

<li>    import_option  </li>

</ul>

<h1>Custom size map</h1>

\image html blsurf_parameters_sizemap.png

User sizes can be defined on faces, edges or vertices.

<ul>

<li>The faces, edges and vertices can belong to the meshed geometrical

object or to its sub-shapes (created using <b>Explode</b> command).</li>

<li>Groups of faces, edges and vertices are also handled.</li>

<li>It is possible to attribute the same size to several geometries using multi-selection.</li>

<li>The sizes are constant values or python functions.</li>

<li>In case of a python function, the following rules must be respected:

<ul>

<li>The name of the function is f.</li>

<li>If geometry is a face or a group of faces, the function is f(u,v).</li>

<li>If geometry is an edge or a group of edges, the function is f(t).</li>

<li>If geometry is a vertex or a group of vertices, the function is f().</li>

<li>The function must return a double.</li>

</ul></li>

</ul>

<br><b>See Also</b> a sample TUI Script of the \ref tui_blsurf "creation of a BLSurf hypothesis", including size map.

\anchor blsurf_sizemap_computation

<h2>Computation of the physical size</h2>

\n

The physical size is obtained by querying sizemap functions associated to the input CAD object for surfaces, curves and points.

Each function can either return a value h (which is then trimmed

between the two bounds hphymin and hphymax), or "no answer" (by not

assigning a value to h), thus providing great flexibility in the

specification of the sizes. The computation depends on whether point P is internal to a surface, internal to a curve, or at the end of several curves:

<ul>

<li> If point P is internal to a surface, the CAD surface size function is queried. If no answer is returned, one interpolates with the values at the vertices of the discretized interface curves.</li>

<li> If point P is internal to a curve, the CAD curve size function is queried first. If no answer is returned, the surface size function is queried for every adjacent surface and the mean value of the returned values is computed. If no answer is returned, sizes h1 and h2 at both ends of the curve are considered (see next item) and the interpolated value is computed.</li>

<li> If point P is at the extremity of several curves, the CAD point size function is queried first. If no answer is returned, the curve size function is queried for every adjacent curve and the mean value of the returned values is computed. If no answer is returned, the surface size function is queried for every adjacent surface and the mean value of the returned values is computed. If there is still no answer returned, the default value hphydef is kept.</li>

</ul>

In order to compute the mean of several values, the arithmetic mean is used by default, but this can be modified by the parameter \ref blsurf_hmean_flag "hmean flag". In the same way, in order to interpolate two values, a linear interpolation is used by default, but this can be modified by \ref blsurf_hinterpol_flag "hinterpol flag".

<h1>Custom enforced vertices</h1>

\image html blsurf_parameters_enforced_vertices.png

It is possible to define some enforced vertices to BLSurf algorithm

without creating any vertices by CAD algorithms.

<ul>

<li>The enforced vertex is the projection of a point defined by its

(x,y,z) coordinates on the selected face.</li>

<li>It is possible to define several enforced vertices on a face or a group of faces.</li>

<li>If the projected point is on the boundary or outside of the face, it will be ignored.</li>

</ul>

<br><b>See Also</b> a sample TUI Script of the \ref tui_blsurf "creation of a BLSurf hypothesis", including enforced vertices.

<h1>Limitations</h1>

Currently BLSURF plugin has the following limitations.

<ul>

<li>The created mesh will contain inverted elements if it is based on a shape,

consisting of more than one face (box, cone, torus...) and if

the option "Allow Quadrangles (Test)" has been checked before

computation.</li>

<li>SIGFPE exception is raised at the attempt to compute the mesh

based on a box when the option "Patch independent" is checked.</li>

<li>BLSURF algorithm cannot be used as a local algorithm (on

sub-meshes) or as a provider of a low-level

mesh for some 3D algorithms, because the BLSURF mesher (and

consequently plugin) does not provide the information on node

parameters on edges (U) and faces (U,V). For example the

following combinations of algorithms are impossible:

<ul>

<li> global MEFISTO or Quadrangle(mapping) + local BLSURF;</li>

<li> BLSUFR + Projection 2D from faces meshed by BLSURF;</li>

<li> local BLSURF + Extrusion 3D;</li>

</ul>

</li>

</ul>

*/